DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

PROJECT REPORT

Unauthorized Mobile Device Detection

using Wi-Fi & BLE Signal Analysis with Machine Learning

Submitted in partial fulfillment of the requirements

for the award of the degree of

Bachelor of Engineering in ELECTRONICS AND COMMUNICATION

Submitted by

[Sachin SB]

Roll No: [24EC0194]

Under the guidance of

[Guide Name], [Designation]

Academic Year 2025 - 2026

CERTIFICATE

This is to certify that the project entitled

"Unauthorized Mobile Device Detection using Wi-Fi & BLE Signal Analysis with Machine Learning"

has been carried out by Sachin SB, Roll No. 24EC0194, in partial fulfillment of the requirements for the award of the degree of Bachelor of Engineering in Computer Science and Engineering during the academic year 2024–2025.

The work is original and has not been submitted elsewhere for any degree or diploma.

| Internal Guide [Guide Name] [Designation] | Head of Department [HOD Name] Dept. of CSE |
| --- | --- |

DECLARATION

We, the undersigned, hereby declare that the project entitled "Unauthorized Mobile Device Detection using Wi-Fi & BLE Signal Analysis with Machine Learning" is our original work submitted to [University/College Name] in partial fulfillment of the requirements for the award of the degree of Bachelor of Engineering in Computer Science and Engineering.

We declare that this project has not been submitted to any other university or institution for the award of any degree, diploma, or any other award. All information furnished in this report is true and complete to the best of our knowledge.

The implementation, experimentation, analysis, and conclusions presented herein are entirely our own work, carried out under the guidance of [Guide Name], [Designation], Department of Computer Science and Engineering.

Name: Sachin SB

Roll No: 24EC0194

Department: Computer Science and Engineering

Date: ___________________

Place: ___________________

ACKNOWLEDGEMENT

We would like to express our sincere gratitude to all those who have contributed to the successful completion of this project.

First and foremost, we extend our heartfelt thanks to our project guide, [Guide Name], [Designation], Department of Computer Science and Engineering, for providing invaluable guidance, constant encouragement, and constructive suggestions throughout the project. Their expertise and dedication have been instrumental in shaping the direction and quality of this work.

We are deeply grateful to [HOD Name], Head of the Department of Computer Science and Engineering, for providing the necessary facilities and for the continuous motivation extended throughout the course of this project.

We would also like to thank the Principal, [College Name], for providing us with an environment conducive to learning and research. Our sincere thanks go to all the faculty members of the Department of CSE for their timely advice and academic support.

We are thankful to our colleagues and friends for their cooperation, discussions, and moral support. Their inputs during the brainstorming sessions helped us identify and overcome various technical challenges.

Finally, we owe immense gratitude to our families for their unwavering support, patience, and encouragement throughout this academic journey. This project would not have been possible without their blessings and motivation.

SACHIN SB

ABSTRACT

The proliferation of mobile devices in modern society has created significant security challenges, particularly in sensitive and restricted environments such as prisons, examination halls, military zones, and corporate boardrooms. Traditional methods of detecting unauthorized wireless devices are often expensive, legally intrusive, or technologically complex. This project presents an innovative, non-intrusive, and cost-effective solution for detecting unauthorized mobile devices by leveraging passive Radio Frequency (RF) signal monitoring combined with Machine Learning (ML) classification.

The proposed system utilizes the ESP32 microcontroller — a dual-mode Wi-Fi and Bluetooth Low Energy (BLE) capable hardware platform — to passively capture wireless signal metadata from the surrounding environment. The system specifically targets Wi-Fi probe requests and BLE advertisement packets, extracting behavioral features such as signal strength (RSSI), signal variation, packet transmission rate, device presence duration, and device authorization status. Critically, no communication content is accessed or intercepted at any point, ensuring full legal compliance and privacy preservation.

Collected data is transmitted via Serial communication to a host computer, where Python-based data collection scripts aggregate the raw logs and derive per-device behavioral feature vectors. A Random Forest Classifier is trained on a labeled dataset comprising three categories: Authorized devices, Suspicious (actively transmitting unauthorized) devices, and Unknown (background or passive) signals. The trained model is serialized using Python's pickle module and deployed for real-time inference.

During detection mode, incoming live signal data from the ESP32 is processed in real-time, features are extracted, and the ML model classifies each detected device as Authorized, Suspicious, or Unknown. Alerts are generated for Suspicious devices, and all detection events are logged to a structured output file. Experimental results demonstrate high classification accuracy, with the Random Forest model effectively distinguishing between authorized and unauthorized device behavioral patterns even in noisy RF environments.

The system presents a practical, deployable solution that is scalable across multiple ESP32 nodes, low-cost relative to commercial alternatives, and free from the legal and ethical complications associated with traditional signal interception methods. This work contributes to the domain of wireless intrusion detection and lays the groundwork for more advanced multi-node localization and deep learning-based detection systems in future iterations.

Keywords: Unauthorized Device Detection, Wi-Fi Probe Requests, BLE Advertisement, ESP32, Machine Learning, Random Forest, RSSI, Passive RF Monitoring, Wireless Intrusion Detection, Signal Behavioral Analysis.

TABLE OF CONTENTS

Certificate	ii

Declaration	iii

Acknowledgement	iv

Abstract	v

Table of Contents	vi

List of Figures	vii

List of Tables	viii

List of Abbreviations	ix

CHAPTER 1 – Introduction	1

1.1 Background and Motivation	1

1.2 Problem Statement	3

1.3 Objectives	4

1.4 Scope of the Project	4

1.5 Organization of the Report	5

CHAPTER 2 – Literature Survey	6

2.1 Existing Systems and Approaches	6

2.2 Wi-Fi Based Detection Systems	7

2.3 Bluetooth-Based Detection Systems	8

2.4 Machine Learning in Wireless Security	9

2.5 Research Gaps Identified	11

CHAPTER 3 – System Design and Methodology	12

3.1 System Overview	12

3.2 Hardware Design	13

3.3 Software Architecture	14

3.4 System Workflow	15

3.5 Block Diagram	17

3.6 Data Flow Diagram	18

CHAPTER 4 – Feature Extraction and Algorithm	19

4.1 Signal Feature Engineering	19

4.2 Random Forest Classifier	21

4.3 Classification Logic	23

4.4 Model Training Process	24

CHAPTER 5 – Implementation	26

5.1 Hardware Setup	26

5.2 ESP32 Firmware	27

5.3 Data Collection Module	31

5.4 Model Training Module	34

5.5 Real-Time Detection Module	37

5.6 Dataset Description	40

CHAPTER 6 – Results and Discussion	42

6.1 Experimental Setup	42

6.2 Model Performance Metrics	43

6.3 Real-Time Detection Output	45

6.4 Discussion	46

CHAPTER 7 – Conclusion and Future Work	48

7.1 Conclusion	48

7.2 Advantages and Limitations	49

7.3 Future Work	50

References	52

Appendix A – Complete Source Code	54

LIST OF FIGURES

Figure 1.1  –  Growth of Mobile Devices Globally (2010–2024)	2

Figure 1.2  –  Security Threats in Restricted Environments	3

Figure 3.1  –  System Architecture Overview	12

Figure 3.2  –  ESP32 Dev Board Pin Layout	13

Figure 3.3  –  System Block Diagram	17

Figure 3.4  –  Data Flow Diagram – Level 0	18

Figure 4.1  –  Feature Extraction Pipeline	20

Figure 4.2  –  Random Forest Decision Tree Structure	22

Figure 4.3  –  Model Training Workflow	25

Figure 5.1  –  Hardware Setup Photograph	26

Figure 5.2  –  Serial Monitor Output from ESP32	30

Figure 5.3  –  Dataset Sample – CSV Structure	40

Figure 5.4  –  Training Data Distribution (Label Counts)	41

Figure 6.1  –  Confusion Matrix – Random Forest Classifier	43

Figure 6.2  –  ROC Curve for Multi-Class Classification	44

Figure 6.3  –  Feature Importance Bar Chart	45

Figure 6.4  –  Real-Time Detection Console Output	46

LIST OF TABLES

Table 2.1  –  Comparison of Existing Detection Methods	11

Table 3.1  –  Hardware Components and Specifications	13

Table 3.2  –  Software Tools and Libraries	14

Table 4.1  –  Feature Description and Relevance	20

Table 5.1  –  Dataset Sample Records	40

Table 5.2  –  Dataset Class Distribution	41

Table 6.1  –  Model Evaluation Metrics	43

Table 6.2  –  Comparison with Baseline Models	44

Table 6.3  –  Detection Output Sample Records	45

LIST OF ABBREVIATIONS

| Abbreviation | Full Form |
| --- | --- |
| BLE | Bluetooth Low Energy |
| DT | Decision Tree |
| ESP32 | Espressif Systems ESP32 Microcontroller |
| IoT | Internet of Things |
| MAC | Media Access Control |
| ML | Machine Learning |
| RF | Radio Frequency |
| RSSI | Received Signal Strength Indicator |
| SDR | Software Defined Radio |
| SVM | Support Vector Machine |
| USB | Universal Serial Bus |
| Wi-Fi | Wireless Fidelity (IEEE 802.11) |
| WIDS | Wireless Intrusion Detection System |

CHAPTER 1
 INTRODUCTION

## 1.1 Background and Motivation

The rapid proliferation of mobile devices over the past two decades has fundamentally transformed global communication and information exchange. According to GSMA Intelligence, the number of unique mobile subscribers worldwide surpassed 5.5 billion in 2023, representing approximately 69% of the global population. While these devices have brought unprecedented convenience and connectivity, they have simultaneously created serious security concerns in environments where wireless communication must be restricted or controlled.

Sensitive facilities — including correctional institutions, military bases, examination centers, boardrooms, and research laboratories — require strict control over wireless devices to prevent unauthorized communication, data leakage, and potential espionage. In Indian prisons alone, the smuggling of mobile phones has been linked to ongoing criminal operations, coordinated gang activities, and even terror-related communications, despite the devices being strictly prohibited. Similarly, examination halls worldwide face persistent challenges with candidates using smartphones to access unauthorized materials during assessments.

The detection of unauthorized wireless devices presents a multifaceted technical challenge. Traditional approaches, such as physical searches and signal jamming, are either manpower-intensive, legally problematic, or technically ineffective. Signal jamming, for instance, is outright illegal in most jurisdictions due to its potential to interfere with legitimate emergency communications. Metal detectors are ineffective against devices that are powered off or placed in signal-blocking pouches. Physical searches are time-consuming and unable to provide continuous monitoring.

Radio Frequency (RF) monitoring offers a more promising avenue. Every wireless-capable mobile device actively or passively emits RF signals that can be captured without intercepting the content of any communication. Wi-Fi enabled devices broadcast probe request frames as they search for known networks. Bluetooth and BLE-enabled devices transmit advertisement packets to announce their presence and capabilities. These metadata-rich signals — which contain no private communication content — can be captured, analyzed, and used to infer the presence and behavior of wireless devices.

The emergence of the ESP32 microcontroller has dramatically lowered the barrier to entry for RF monitoring applications. The ESP32, developed by Espressif Systems, integrates a dual-core processor, Wi-Fi (IEEE 802.11 b/g/n) and Bluetooth 4.2/BLE capabilities, and a rich set of peripherals into a single chip that costs approximately $3–5 USD. This makes it an ideal platform for low-cost, deployable wireless monitoring nodes.

Coupled with advances in Machine Learning, particularly ensemble methods such as Random Forests, it is now possible to train classification models that can distinguish between the behavioral RF signatures of authorized devices (known to the security system) and unauthorized devices (unfamiliar or exhibiting suspicious transmission patterns). This project integrates these three elements — passive RF monitoring, the ESP32 platform, and ML-based classification — into a cohesive wireless intrusion detection system.

## 1.2 Problem Statement

In sensitive environments such as correctional facilities, examination halls, military zones, and secure corporate premises, unauthorized mobile devices pose significant security threats. These devices can be used to:

- Facilitate illegal communications between prisoners and external criminal networks

- Enable examination malpractice through access to digital resources and remote assistance

- Leak classified, confidential, or proprietary information through covert wireless transmission

- Act as remote triggers for malicious activities in military and high-security environments

Existing detection solutions are inadequate for the following reasons:

- Commercial WIDS (Wireless Intrusion Detection Systems) are prohibitively expensive for widespread deployment, often costing thousands of dollars per node

- Signal jamming is illegal under telecommunications laws in most countries and poses risks to emergency communications

- Physical inspection is labor-intensive, periodic rather than continuous, and relies on human vigilance which can be exploited

- Content-based signal interception raises serious legal and ethical concerns under privacy legislation

Therefore, a critical need exists for a system that can detect the presence of unauthorized mobile devices continuously, non-intrusively, at low cost, and without intercepting any communication content. This project addresses that need directly.

## 1.3 Objectives

The primary objectives of this project are:

- To design and implement a passive RF monitoring system using ESP32 hardware that captures Wi-Fi probe requests and BLE advertisement packets from nearby mobile devices without intercepting communication content.

- To develop a robust feature extraction pipeline that derives meaningful behavioral features from raw RF signal data, including RSSI statistics, packet transmission rates, and device presence duration.

- To build, train, and evaluate a Machine Learning classification model (Random Forest Classifier) capable of distinguishing between Authorized, Suspicious, and Unknown wireless devices.

- To implement a real-time detection system that processes live ESP32 sensor data, applies the trained model, and generates actionable alerts for suspicious device detections.

- To evaluate the system's performance in terms of classification accuracy, precision, recall, F1-score, and real-world detection latency.

- To demonstrate the system's scalability, cost-effectiveness, and practical deployability in real-world restricted environments.

## 1.4 Scope of the Project

This project encompasses the following scope:

- Hardware: ESP32 Development Board used as a passive RF sensor node. No additional hardware beyond ESP32, USB cable, and a host PC/laptop is required.

- Wireless Protocols: Wi-Fi 802.11 b/g/n (specifically probe request frames in promiscuous mode) and Bluetooth Low Energy (BLE advertisement packets). 4G/5G cellular detection is outside the current scope.

- Machine Learning: Supervised learning using Random Forest Classifier. The model operates on a pre-defined feature set and requires a labeled training dataset.

- Detection Granularity: Device-level detection (per MAC address), not communication content analysis.

- Output: Real-time console alerts and structured CSV output for logging and audit purposes.

- Environment: Indoor environments with moderate RF noise levels. Performance may vary in very high-density RF environments.

## 1.5 Organization of the Report

The remainder of this report is organized as follows:

- Chapter 2 presents a comprehensive literature survey of existing wireless intrusion detection approaches and related work in RF-based device detection and ML-based wireless security.

- Chapter 3 describes the overall system design, hardware components, software architecture, and system workflow.

- Chapter 4 details the feature extraction methodology and provides an in-depth explanation of the Random Forest classifier and classification logic.

- Chapter 5 covers the full implementation, including ESP32 firmware code, Python scripts for data collection, model training, and real-time detection.

- Chapter 6 presents experimental results, model performance metrics, and a discussion of findings.

- Chapter 7 concludes the report with a summary of achievements, identified limitations, and directions for future work.

- References and Appendices follow at the end of the report.

CHAPTER 2
 LITERATURE SURVEY

## 2.1 Existing Systems and Approaches

The detection of unauthorized wireless devices has been an active area of research since the early 2000s, driven primarily by the increasing prevalence of Wi-Fi networks and the associated security challenges. Early approaches focused almost exclusively on network-layer anomaly detection within existing infrastructure, gradually evolving toward more sophisticated RF-layer monitoring as hardware costs decreased and computational capabilities improved.

Intrusion Detection Systems (IDS) in the wireless domain — termed Wireless Intrusion Detection Systems (WIDS) — have historically operated by monitoring 802.11 management frames for anomalous patterns indicative of rogue access points, deauthentication attacks, or unauthorized client associations. Commercial systems such as Cisco Adaptive Wireless IDS, Aruba RFProtect, and AirMagnet Enterprise represent the state of the art in enterprise WIDS deployment, offering features including rogue AP detection, client blacklisting, over-the-air packet capture, and forensic analysis. However, these systems are designed for corporate network security rather than physical device detection in non-networked environments, and their costs — often $10,000 or more per deployment — make them inaccessible for cost-sensitive applications.

The academic literature offers a rich foundation. Bahl and Padmanabhan (2000) demonstrated that RSSI measurements from 802.11 access points could be used for indoor positioning with room-level accuracy, establishing the foundational relationship between RSSI and physical device presence that underlies much subsequent work. Milner et al. (2012) extended this concept to rogue device detection in enterprise environments, using RSSI triangulation from multiple sensors to localize unauthorized devices. Their work highlighted both the potential and the limitations of purely RSSI-based approaches — particularly the susceptibility to multipath fading and environmental interference.

## 2.2 Wi-Fi Based Detection Systems

Wi-Fi probe requests represent a uniquely valuable signal source for passive device detection. When a Wi-Fi enabled device is not associated with any network, its firmware periodically broadcasts probe request frames on multiple channels, advertising the SSIDs of previously connected networks or sending null-probe requests to solicit responses from any available access point. These frames are transmitted even when the user is not actively using the device, making them a reliable indicator of physical device presence.

Franklin et al. (2006) were among the first to systematically study probe request behavior, documenting the transmission characteristics of various device types and operating systems. They found significant variation in probe rates (ranging from 1 to 40 probes per minute depending on device state), SSID list lengths, and inter-probe intervals — variations that could potentially be exploited for device fingerprinting.

Pang et al. (2007) introduced the concept of using probe requests for passive device presence detection without network infrastructure, demonstrating the feasibility of using a single monitoring node to detect devices at ranges up to 50 meters in open environments. Their work is directly relevant to the current project, as it validates the ESP32's use case as a standalone probe request monitor.

More recent work by Vanhoef et al. (2016) raised significant concerns about probe request-based tracking, demonstrating that unique probe request patterns could be used to track individuals across physical locations. This observation, while a privacy concern in other contexts, is precisely the property exploited in this project for security purposes — the behavioral uniqueness of probe request patterns enables reliable device classification.

The introduction of MAC address randomization in iOS 8 (2014) and Android 6.0 (2015) complicates probe-request-based detection by periodically changing the source MAC address. However, research by Martin et al. (2017) and Matte et al. (2016) has demonstrated that randomization is inconsistently implemented across device manufacturers and can be circumvented through analysis of information elements within probe request frames, RSSI fingerprinting, and temporal correlation of probe sequences.

## 2.3 Bluetooth-Based Detection Systems

Bluetooth Low Energy (BLE), introduced in Bluetooth 4.0 (2010), operates on 40 channels in the 2.4 GHz band, with three dedicated advertising channels (37, 38, and 39) used for device discovery and advertisement broadcasting. BLE-capable devices in an undiscovered state periodically transmit advertisement packets on these channels, containing a device identifier (either a static or random MAC address) and optional payload data.

The passive nature of BLE advertisement scanning makes it particularly well-suited for unauthorized device detection. Unlike active Bluetooth inquiry scans, passive BLE scanning requires no interaction with target devices and is completely invisible to the monitored devices. The ESP32's BLE stack supports passive advertisement scanning with configurable scan windows and intervals, enabling efficient coverage of all three advertising channels.

Singh et al. (2020) demonstrated a BLE-based personnel tracking system in indoor environments, achieving room-level localization accuracy using RSSI measurements from multiple ESP32 nodes. Their work validates the use of ESP32 as a BLE monitoring platform and provides baseline RSSI measurement accuracy figures that are relevant to the feature engineering in this project.

Cominelli et al. (2020) conducted a comprehensive study of BLE advertisement packet characteristics across over 150 different device models, identifying significant behavioral differences between device categories (smartphones, wearables, IoT devices) that could be exploited for device type classification. Their feature set — which includes advertising interval, payload length, manufacturer-specific data presence, and TX power levels — partially overlaps with the feature engineering approach in this project.

## 2.4 Machine Learning in Wireless Security

The application of Machine Learning to wireless security has grown substantially over the past decade, driven by the increasing complexity of wireless environments and the limitations of signature-based detection approaches. ML-based approaches offer the advantage of learning normal behavioral patterns from data and flagging anomalies without requiring explicit rule definition.

Supervised classification algorithms have been particularly well-studied for wireless intrusion detection. Breiman's Random Forest algorithm (2001) has emerged as a consistently high-performing choice for this domain due to its robustness to noise, ability to handle non-linear feature relationships, built-in feature importance estimation, and resistance to overfitting through ensemble averaging.

Yang et al. (2018) applied Random Forest classification to Wi-Fi network traffic analysis, achieving 97.3% accuracy in detecting eight categories of wireless attacks including deauthentication, probe flooding, and rogue AP injection. Their work demonstrates that RF-based behavioral features — particularly packet rate statistics and temporal transmission patterns — are highly discriminative for security classification tasks.

Aminanto and Kim (2016) compared multiple ML algorithms for Wi-Fi intrusion detection, including SVM, Neural Networks, k-NN, Naive Bayes, and Random Forest. Random Forest consistently achieved the highest F1-scores across multiple attack categories, with particular strength in handling imbalanced class distributions — a property valuable in the current project where suspicious device detections are expected to be less frequent than authorized device observations.

Deep learning approaches have also been explored. Doriguzzi-Corin et al. (2020) applied Convolutional Neural Networks (CNN) to packet-level features for wireless anomaly detection, achieving state-of-the-art accuracy but requiring significantly larger training datasets and computational resources than traditional ensemble methods. For the constrained setting of this project — small initial datasets, real-time inference requirements, and limited computational resources — the Random Forest approach represents a better-suited choice.

Regarding feature engineering specifically for device classification from RSSI data, Mirowski et al. (2011) demonstrated that temporal RSSI statistics (mean, variance, minimum, maximum, and autocorrelation) are the most informative features for location and device classification tasks. This finding directly informs the feature set design in this project, where RSSI mean and standard deviation are included as primary features.

## 2.5 Research Gaps Identified

Despite the extensive research surveyed above, several gaps remain that this project addresses:

- Integration Gap: Existing systems typically focus on either Wi-Fi or Bluetooth detection, rarely integrating both signal types into a unified detection pipeline. This project combines both protocols to maximize detection coverage.

- Cost Gap: Commercial WIDS solutions are prohibitively expensive. This project demonstrates that an effective detection system can be built for under $10 in hardware cost using the ESP32 platform.

- Non-Intrusive Gap: Many existing academic approaches require network infrastructure (access points, existing network frameworks), or involve active scanning that reveals the monitoring system's presence. This project operates entirely passively.

- Real-Time Gap: Most academic work focuses on offline analysis of captured data. This project implements a complete pipeline from hardware signal capture through real-time ML inference.

| Approach | Cost | Protocol Coverage | Real-Time | Non-Intrusive | ML-Based |
| --- | --- | --- | --- | --- | --- |
| Commercial WIDS | Very High ($$$) | Wi-Fi only | Yes | Partial | Limited |
| Signal Jamming | Medium | All RF | Yes | No (illegal) | No |
| Physical Search | Low | All | No | Yes | No |
| Academic RF Monitor | Medium | Single protocol | No | Yes | Partial |
| This Project (ESP32+ML) | Very Low ($) | Wi-Fi + BLE | Yes | Yes | Yes |

Table 2.1: Comparison of Existing Detection Methods

CHAPTER 3
 SYSTEM DESIGN AND METHODOLOGY

## 3.1 System Overview

The proposed system is a passive wireless intrusion detection system (WIDS) that leverages the dual RF capabilities of the ESP32 microcontroller to capture Wi-Fi probe requests and BLE advertisement packets from the surrounding environment. The captured signal metadata is processed through a Python-based pipeline that extracts behavioral features per device, classifies each device using a trained Random Forest model, and generates real-time alerts for unauthorized or suspicious device detections.

The system operates entirely on signal metadata — specifically, MAC addresses, signal strength (RSSI), packet types, and timestamps. No packet payloads are decoded, no communication content is accessed, and no decryption is attempted at any stage. This ensures that the system remains compliant with telecommunications privacy laws while still providing effective detection capabilities.

The architecture follows a three-tier design pattern: (1) the ESP32 sensor tier, which handles raw RF signal capture; (2) the processing tier, which runs on the host computer and handles feature extraction, model training, and real-time inference; and (3) the output tier, which provides alerts and structured logging. Each tier is modular and independently replaceable, facilitating future enhancements such as cloud integration or multi-node deployment.

## 3.2 Hardware Design

The hardware component of the system consists of a single ESP32 Development Board connected to a host computer via USB. The ESP32 is selected for the following technical reasons:

- Dual-mode wireless: Integrated 2.4 GHz Wi-Fi (IEEE 802.11 b/g/n) and Bluetooth 4.2 with BLE support eliminate the need for separate hardware for each protocol.

- Promiscuous mode support: The ESP32's Wi-Fi driver supports promiscuous mode, enabling capture of all 802.11 frames including probe requests from non-associated devices.

- BLE passive scanning: The ESP32's BLE stack supports passive advertisement scanning without transmitting any packets.

- Cost: At $3–5 USD per unit, the ESP32 is orders of magnitude cheaper than commercial monitoring hardware.

- Ecosystem: Rich support in Arduino IDE and ESP-IDF SDK with extensive community documentation and libraries.

| Component | Specification | Purpose |
| --- | --- | --- |
| ESP32 Dev Board | Espressif ESP32-WROOM-32, Dual-core 240 MHz, 4MB Flash | RF signal capture node |
| Wi-Fi Chipset | IEEE 802.11 b/g/n, 2.4 GHz, 150 Mbps max | Probe request capture (promiscuous mode) |
| BLE Chipset | Bluetooth 4.2 / BLE 4.2, 2 Mbps PHY | BLE advertisement scanning (passive) |
| USB Interface | CP2102 or CH340 USB-to-UART bridge | Serial data transfer to host PC |
| Antenna | Integrated PCB antenna, 2 dBi gain | RF signal reception |
| Host Computer | Any PC/Laptop with USB port | Data processing and ML inference |

Table 3.1: Hardware Components and Specifications

## 3.3 Software Architecture

The software stack is entirely Python-based on the host side, with Arduino C++ firmware on the ESP32. The modular structure ensures that individual components can be updated or replaced independently.

| Component | Technology | Purpose |
| --- | --- | --- |
| ESP32 Firmware | Arduino C++ (esp32_wifi_ble_scan.ino) | Capture and serialize Wi-Fi + BLE signals |
| Serial Interface | PySerial (Python) | Read raw data from ESP32 via USB |
| Data Collection | Python + Pandas (data_collection.py) | Aggregate raw logs into feature vectors |
| Feature Engineering | NumPy + Pandas | Compute RSSI stats, packet rates, duration |
| Model Training | scikit-learn Random Forest (train_model.py) | Train and serialize ML classifier |
| Real-Time Detection | Python + scikit-learn (detect.py) | Load model, classify live data, alert |
| Output Logging | Pandas CSV writer | Store detection events to CSV |
| Development IDE | Arduino IDE + VS Code + Python 3.x | Code editing and debugging |

Table 3.2: Software Tools and Libraries

## 3.4 System Workflow

The end-to-end operational workflow of the system proceeds through seven sequential phases, each building on the outputs of the previous. The following subsections describe each phase in detail.

### Phase 1: Signal Capture

The ESP32 is initialized in a dual-mode state: Wi-Fi is configured in promiscuous mode on a scanning channel, while BLE is initialized with a passive scanner. A custom promiscuous packet handler (callback function registered via esp_wifi_set_promiscuous_rx_cb()) processes each captured Wi-Fi frame, filtering for management frames of subtype Probe Request. For BLE, the gap_event_handler() processes BLE_GAP_EVENT_DISC events to extract advertisement packets. From each captured frame, the following fields are extracted and serialized to the UART output: packet type (WIFI or BLE), source MAC address, RSSI, and Unix timestamp. The serial output format is: TYPE,MAC_ADDRESS,RSSI,TIMESTAMP.

### Phase 2: Data Transfer

The ESP32 connects to the host computer via USB. The CP2102 or CH340 USB-to-UART bridge chip on the ESP32 development board presents a virtual serial port to the host OS (typically /dev/ttyUSB0 on Linux, COM3 on Windows). PySerial is used to open this port at 115200 baud (default ESP32 serial speed) and read lines of text data continuously.

### Phase 3: Data Collection and Aggregation

The data_collection.py script reads the serial stream, parses each CSV-formatted line, and builds a per-device observation log indexed by MAC address. The raw log entries are timestamped and stored in an in-memory DataFrame. Observations are periodically aggregated over a configurable window (default: 60 seconds), and the resulting per-device feature records are appended to the training dataset CSV. During data collection, the operator labels each device as Authorized (0), Suspicious (1), or Unknown (2), enabling supervised model training.

### Phase 4: Feature Extraction

From the raw per-device observation window, five behavioral features are computed. These features are designed to capture the temporal and spatial transmission behavior of each device rather than any communication content. The feature computation is performed using NumPy array operations for computational efficiency.

### Phase 5: Model Training

The train_model.py script loads the training_data.csv dataset, splits it into training and test sets using stratified random sampling (80/20 split), trains a Random Forest Classifier using scikit-learn's RandomForestClassifier with configured hyperparameters, evaluates the model on the test set, and serializes the fitted model to model.pkl using Python's pickle module. Feature importance scores are also computed and logged during this phase.

### Phase 6: Real-Time Detection

The detect.py script loads the serialized model from model.pkl and opens the serial port to the ESP32. It processes incoming signal data in real-time using the same feature extraction logic as the training pipeline. For each device observation window that completes, the feature vector is passed to the loaded model's predict() method. The resulting class label (0, 1, or 2) is mapped to the corresponding human-readable label (Authorized, Suspicious, or Unknown). If the predicted label is Suspicious, an alert is printed to the console with device details.

### Phase 7: Output and Logging

All detection events are appended to detection_output.csv, a structured log file with columns: timestamp, mac_address, rssi_mean, rssi_std, packet_rate, duration, known, predicted_label, alert_flag. This log provides an audit trail of all detected devices and can be reviewed post-session for forensic analysis or system calibration.

## 3.5 Block Diagram

The following block diagram illustrates the high-level system architecture and signal flow from physical RF environment through to detection output.

[ PHYSICAL ENVIRONMENT ] Mobile Devices (Authorized / Unauthorized / Unknown) ↓ Wi-Fi Probe Requests + BLE Advertisements (2.4 GHz RF) [ ESP32 SENSOR NODE ] Promiscuous Wi-Fi Scan + Passive BLE Scan → TYPE,MAC,RSSI,TIMESTAMP ↓ USB Serial (115200 baud) [ HOST COMPUTER — DATA COLLECTION ] PySerial → data_collection.py → Feature Extraction → training_data.csv ↓ Supervised Training [ ML MODEL — RANDOM FOREST CLASSIFIER ] train_model.py → model.pkl (serialized) ↓ Real-Time Inference [ DETECTION ENGINE ] detect.py → Authorized / Suspicious / Unknown → Alert + detection_output.csv

Figure 3.1: System Block Diagram

CHAPTER 4
 FEATURE EXTRACTION AND ALGORITHM

## 4.1 Signal Feature Engineering

Feature engineering is the process of transforming raw sensor data into a structured representation that enables Machine Learning algorithms to learn discriminative patterns. In this project, raw RF signal data — consisting of individual packet events tagged with MAC address, RSSI, type, and timestamp — must be aggregated and transformed into per-device feature vectors before classification.

The aggregation window is a critical parameter: it defines the time period over which individual packet events are grouped for a single device before computing features. A window that is too short (< 10 seconds) may not capture enough packets for reliable statistics, while a window that is too long (> 5 minutes) introduces detection latency. Based on empirical testing, a 60-second aggregation window provides a good balance between statistical reliability and detection speed.

| Feature | Formula / Derivation | Relevance to Detection |
| --- | --- | --- |
| rssi_mean | Mean of all RSSI values in window: μ = Σ(RSSI_i) / N | Indicates device proximity; authorized devices at known positions have expected RSSI ranges |
| rssi_std | Standard deviation: σ = sqrt(Σ(RSSI_i - μ)² / N) | High std indicates mobility or multipath fading; distinguishes active from static devices |
| packet_rate | Count of packets in window / window duration (seconds) | Suspicious devices often have elevated packet rates due to active searching behavior |
| duration | Timestamp of last packet - timestamp of first packet in window | Short duration may indicate unauthorized transient devices; long duration indicates stationed devices |
| known | Binary flag: 1 if MAC in authorized whitelist, 0 otherwise | Primary discriminator between authorized and unauthorized devices |

Table 4.1: Feature Description and Relevance to Detection

The rssi_mean feature captures the average signal strength of a device over the observation window. Since signal strength is inversely related to distance (via the path loss equation), devices positioned within a facility at known, authorized locations will tend to have predictable RSSI ranges as measured by a fixed sensor node. A device appearing at an unexpected signal strength (either too strong — suggesting it is very close, possibly concealed, or too weak — suggesting it is at the perimeter) can be flagged as potentially suspicious.

The rssi_std feature measures the variability of signal strength over time. A stationary device will exhibit relatively low RSSI variation as the RF channel remains relatively stable. A device that is moving (e.g., being carried by a person) or that is intermittently obstructed will show higher RSSI variance. Active unauthorized devices that are being moved around to avoid detection would be expected to exhibit elevated RSSI standard deviations.

The packet_rate feature is particularly important for distinguishing active smartphones (which probe aggressively while screen-on and searching for networks) from passive or powered-off devices. Modern smartphones in an active, screen-on state may transmit 5–20 probe requests per minute on each Wi-Fi channel, while devices in a sleep state may probe only once every few minutes or not at all. An unusually high packet rate is a strong indicator of active device usage.

The duration feature helps distinguish incidental signal captures (devices passing by briefly, such as in a corridor outside a monitored area) from persistent unauthorized presences (a device that has been within range for an extended period). A device detected repeatedly across multiple observation windows with consistent characteristics is a stronger candidate for classification as either authorized or suspicious.

The known flag is a binary feature derived from a pre-configured whitelist of authorized device MAC addresses. In a real deployment, this whitelist would be populated with the MAC addresses of all legitimate devices belonging to authorized personnel (guards, administrators, medical staff, etc.). A device with a MAC address not in the whitelist receives a known value of 0, making it a candidate for Suspicious or Unknown classification.

## 4.2 Random Forest Classifier

The Random Forest algorithm, introduced by Leo Breiman in 2001, is an ensemble learning method that operates by constructing a large number of Decision Trees during training and outputting the mode of the class probabilities of the individual trees for classification tasks. The key innovations of Random Forest over standard bagged Decision Trees are: (1) feature randomness — at each node split, a random subset of features is considered rather than all features, reducing tree correlation; and (2) bootstrap aggregation (bagging) — each tree is trained on a bootstrap sample of the training data, introducing data diversity.

These mechanisms work together to create an ensemble that is significantly more robust to overfitting than individual decision trees, handles non-linear feature relationships naturally (as individual trees are non-parametric), and produces reliable probability estimates alongside hard class predictions. Furthermore, Random Forest provides built-in feature importance scores based on the mean decrease in impurity across all trees and all splits involving each feature, which serves as a valuable diagnostic tool.

In scikit-learn, the RandomForestClassifier is configured with the following key hyperparameters for this project:

- n_estimators = 100: 100 decision trees are grown. This number provides sufficient ensemble stability while keeping training time manageable.

- max_depth = None: Trees are grown to full depth, maximizing individual tree expressiveness. Overfitting is controlled by the ensemble mechanism rather than individual tree depth limits.

- min_samples_split = 2: Minimum samples required to split an internal node. The default setting is appropriate for the feature-rich low-sample regime.

- class_weight = 'balanced': Adjusts class weights inversely proportional to class frequencies. This is critical for handling potential class imbalance where Authorized records may outnumber Suspicious records.

- random_state = 42: Fixed seed for reproducibility of training results.

The algorithmic steps of Random Forest training are as follows:

- For each tree t = 1 to n_estimators: Sample N instances with replacement from the training dataset (bootstrap sample).

- Grow a Decision Tree on the bootstrap sample: at each node, randomly select k features (default k = sqrt(total features)) and find the best split among these k features using Gini impurity as the splitting criterion.

- Continue growing the tree until no further splits are possible (all leaf nodes are pure or contain fewer than min_samples_split samples).

- For prediction on a new instance x: Pass x through all 100 trees, collect the predicted class from each tree, and output the majority vote as the final prediction.

- For probability estimation: Average the class probability estimates from all 100 trees.

## 4.3 Classification Logic

The classification logic follows a hierarchical decision process that mirrors how a human security analyst would reason about a detected device. The primary discriminator is the known flag: a device with a registered MAC address is provisionally classified as Authorized regardless of its transmission behavior, though significant behavioral anomalies (very high packet rate, unexpected RSSI) would still be flagged for review.

For unknown devices (known = 0), the ML model applies the learned behavioral patterns from training data to classify the device. The classification boundaries are learned automatically from data but correspond intuitively to the following logic:

Classification Decision Logic (conceptual):

IF known == 1:

→ Predicted Label: AUTHORIZED

(Device is in the whitelist)

ELSE (known == 0):

IF packet_rate > threshold_high AND rssi_mean > threshold_rssi:

→ Predicted Label: SUSPICIOUS

(Active device, close proximity, unknown identity)

ELIF packet_rate < threshold_low AND duration < threshold_short:

→ Predicted Label: UNKNOWN

(Weak signal, transient, likely background noise)

ELSE:

→ Predicted Label: SUSPICIOUS or UNKNOWN

(Determined by RF model probability estimate)

It is important to note that the thresholds above are illustrative. The actual classification boundaries are not hardcoded but are learned by the Random Forest from the training data distribution. This makes the system adaptive: if the typical packet rate of suspicious devices in a particular environment is different from general assumptions, the model learns this from environment-specific training data.

## 4.4 Model Training Process

The model training process is designed to be repeatable and environment-specific. Rather than providing a single globally trained model, the system architecture encourages operators to collect training data in the specific environment where the system will be deployed. This is important because RF characteristics (RSSI distributions, channel interference levels, typical device densities) vary significantly between environments.

The training workflow begins with a data collection phase during which the operator uses data_collection.py to capture signal data from known device categories. Authorized devices (belonging to security staff and administrators) are logged during a controlled session where their MAC addresses are recorded and labeled 0. Suspicious device behavior is simulated by using smartphones in active use (screen on, actively searching for Wi-Fi networks) with unknown MAC addresses and labeling them 1. Unknown signals are captured passively from any devices detected in the environment that are neither actively authorized nor actively simulated suspicious, labeled 2.

The resulting training_data.csv typically contains 500–2000 records across the three classes, depending on the duration of data collection and the number of devices in the environment. The 80/20 stratified train-test split ensures that all three classes are proportionally represented in both the training and evaluation subsets.

After training, the model is evaluated using the following metrics: Overall Accuracy, per-class Precision, per-class Recall, per-class F1-score, and Confusion Matrix. Feature importance scores are computed and logged to help operators understand which features are most discriminative in their specific deployment environment, potentially guiding future feature engineering improvements.

CHAPTER 5
 IMPLEMENTATION

## 5.1 Hardware Setup

The hardware setup for this project is intentionally minimal to demonstrate the feasibility of low-cost wireless monitoring. The primary component — the ESP32 Development Board — is connected to the host computer via a standard Micro-USB or USB-C cable. No external power supply, additional sensors, or peripheral hardware is required.

In a real deployment scenario, multiple ESP32 nodes would be placed at strategic points within the monitored environment (entry points, corridor intersections, room corners) to maximize coverage and enable potential future multi-point localization. However, even a single node provides effective area coverage with a detection radius of approximately 30–50 meters in open indoor environments and 10–20 meters through multiple walls, depending on wall material and ESP32 antenna orientation.

Before flashing the firmware, the ESP32 board must be set up in the Arduino IDE with the ESP32 board support package installed (via Boards Manager → https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json). The board is then selected as "ESP32 Dev Module" with default settings. The USB driver for the onboard USB-to-UART bridge chip (CP2102 or CH340, depending on board variant) must be installed on the host computer.

## 5.2 ESP32 Firmware

The ESP32 firmware (esp32_wifi_ble_scan.ino) is the core hardware interface layer. It initializes both the Wi-Fi subsystem in promiscuous mode and the BLE subsystem with passive scanning enabled, then continuously captures and serializes signal data to the USB serial port.

5.2.1 Complete Firmware Code

#include <Arduino.h>

#include "esp_wifi.h"

#include "esp_bt.h"

#include "esp_gap_ble_api.h"

#include "nvs_flash.h"

// ─── Wi-Fi Promiscuous Callback ─────────────────────────────────────────────

void wifi_sniffer_packet_handler(void *buf, wifi_promiscuous_pkt_type_t type) {

if (type != WIFI_PKT_MGMT) return;  // Only process management frames

wifi_promiscuous_pkt_t *pkt = (wifi_promiscuous_pkt_t *)buf;

uint8_t *payload = pkt->payload;

// Check for Probe Request subtype (FC byte 1 bits 2-5 = 0100)

uint8_t frame_ctrl_byte = payload[0];

uint8_t subtype = (frame_ctrl_byte >> 4) & 0x0F;

if (subtype != 0x04) return;  // 0x04 = Probe Request

// Extract source MAC address (bytes 10-15 in 802.11 header)

uint8_t *mac = &payload[10];

int8_t rssi = pkt->rx_ctrl.rssi;

unsigned long ts = millis() / 1000;

Serial.printf("WIFI,%02X:%02X:%02X:%02X:%02X:%02X,%d,%lu\n",

mac[0],mac[1],mac[2],mac[3],mac[4],mac[5], rssi, ts);

}

// ─── BLE GAP Event Callback ─────────────────────────────────────────────────

void ble_gap_event_handler(struct ble_gap_event *event, void *arg) {

if (event->type != BLE_GAP_EVENT_DISC) return;

uint8_t *mac = event->disc.addr.val;

int8_t rssi = event->disc.rssi;

unsigned long ts = millis() / 1000;

Serial.printf("BLE,%02X:%02X:%02X:%02X:%02X:%02X,%d,%lu\n",

mac[5],mac[4],mac[3],mac[2],mac[1],mac[0], rssi, ts);

}

// ─── Setup ───────────────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

nvs_flash_init();

// Initialize Wi-Fi in promiscuous mode

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

esp_wifi_init(&cfg);

esp_wifi_set_mode(WIFI_MODE_NULL);

esp_wifi_start();

esp_wifi_set_promiscuous(true);

esp_wifi_set_promiscuous_rx_cb(wifi_sniffer_packet_handler);

// Initialize BLE passive scan

esp_bt_controller_config_t bt_cfg = BT_CONTROLLER_INIT_CONFIG_DEFAULT();

esp_bt_controller_init(&bt_cfg);

esp_bt_controller_enable(ESP_BT_MODE_BLE);

esp_bluedroid_init();

esp_bluedroid_enable();

// Register GAP callback and start passive BLE scan

esp_ble_gap_register_callback(ble_gap_event_handler);

esp_ble_scan_params_t scan_params = {

.scan_type = BLE_SCAN_TYPE_PASSIVE,

.own_addr_type = BLE_ADDR_TYPE_PUBLIC,

.scan_filter_policy = BLE_SCAN_FILTER_ALLOW_ALL,

.scan_interval = 0x0050,  // 50ms

.scan_window   = 0x0030,  // 30ms

};

esp_ble_gap_set_scan_params(&scan_params);

esp_ble_gap_start_scanning(0);  // 0 = scan indefinitely

Serial.println("ESP32 Scanner Initialized. Listening...");

}

// ─── Loop ────────────────────────────────────────────────────────────────────

void loop() {

// Rotate Wi-Fi scan channels (1-13) every 200ms for broader coverage

static uint8_t channel = 1;

esp_wifi_set_channel(channel, WIFI_SECOND_CHAN_NONE);

channel = (channel % 13) + 1;

delay(200);

}

The firmware operates at 115200 baud and continuously outputs data lines to the USB serial port. The channel rotation in the main loop ensures coverage of all 13 Wi-Fi channels (channels 1, 6, and 11 are the non-overlapping channels for 802.11 b/g/n, but devices probe on all channels) at 200ms intervals, providing a complete sweep every 2.6 seconds.

## 5.3 Data Collection Module

The data collection module (data_collection.py) bridges the hardware sensor and the ML pipeline. It reads the serial stream from the ESP32, parses the raw CSV lines, maintains a per-device observation buffer, and periodically flushes feature-extracted records to the training CSV with operator-supplied labels.

# data_collection.py — Data Collection and Feature Extraction Module

import serial

import pandas as pd

import numpy as np

import time

import csv

from collections import defaultdict

from datetime import datetime

# ─── Configuration ──────────────────────────────────────────────────────────

SERIAL_PORT = '/dev/ttyUSB0'   # Windows: 'COM3'

BAUD_RATE   = 115200

WINDOW_SEC  = 60               # Feature aggregation window in seconds

OUTPUT_CSV  = 'training_data.csv'

# Authorized device MAC addresses (whitelist)

AUTHORIZED_MACS = {

'AA:BB:CC:DD:EE:FF',  # Security guard device

'11:22:33:44:55:66',  # Admin laptop

'00:11:22:33:44:55',  # Authorized tablet

}

# ─── Feature Extraction ──────────────────────────────────────────────────────

def extract_features(records, mac):

"""Compute per-device feature vector from list of (rssi, timestamp) tuples."""

if len(records) < 2:

return None  # Insufficient data for reliable feature extraction

rssi_values = [r[0] for r in records]

timestamps  = [r[1] for r in records]

rssi_mean   = float(np.mean(rssi_values))

rssi_std    = float(np.std(rssi_values))

duration    = float(max(timestamps) - min(timestamps))

packet_rate = len(records) / max(duration, 1.0)  # packets per second

known       = 1 if mac in AUTHORIZED_MACS else 0

return {

'rssi_mean':   round(rssi_mean, 2),

'rssi_std':    round(rssi_std, 2),

'packet_rate': round(packet_rate, 4),

'duration':    round(duration, 2),

'known':       known,

}

# ─── Main Collection Loop ────────────────────────────────────────────────────

def collect(label):

"""Collect data from ESP32 serial stream for a specified label.

label: 0=Authorized, 1=Suspicious, 2=Unknown"""

ser = serial.Serial(SERIAL_PORT, BAUD_RATE, timeout=1)

device_buffer = defaultdict(list)  # mac -> [(rssi, timestamp), ...]

window_start  = time.time()

print(f'Collecting label={label} data. Press Ctrl+C to stop.')

try:

while True:

line = ser.readline().decode('utf-8', errors='ignore').strip()

if not line or ',' not in line:

continue

parts = line.split(',')

if len(parts) != 4:

continue

pkt_type, mac, rssi_str, ts_str = parts

try:

rssi = int(rssi_str)

ts   = float(ts_str)

except ValueError:

continue

device_buffer[mac].append((rssi, ts))

# Flush window

if time.time() - window_start >= WINDOW_SEC:

with open(OUTPUT_CSV, 'a', newline='') as f:

writer = csv.DictWriter(f, fieldnames=[

'rssi_mean','rssi_std','packet_rate','duration','known','label'])

for m, recs in device_buffer.items():

feat = extract_features(recs, m)

if feat:

feat['label'] = label

writer.writerow(feat)

device_buffer.clear()

window_start = time.time()

print(f'[{datetime.now()}] Window flushed.')

except KeyboardInterrupt:

ser.close()

print('Collection stopped.')

## 5.4 Model Training Module

The model training module (train_model.py) loads the collected training dataset, performs preprocessing, trains the Random Forest classifier, evaluates its performance on a held-out test set, and saves the fitted model to disk for use by the real-time detection module.

# train_model.py — Random Forest Model Training Module

import pandas as pd

import numpy as np

import pickle

from sklearn.ensemble import RandomForestClassifier

from sklearn.model_selection import train_test_split, cross_val_score

from sklearn.metrics import (classification_report, confusion_matrix,

accuracy_score, roc_auc_score)

from sklearn.preprocessing import label_binarize

import warnings

warnings.filterwarnings('ignore')

# ─── Load and Validate Dataset ───────────────────────────────────────────────

df = pd.read_csv('training_data.csv')

print(f'Dataset loaded: {len(df)} records')

print(f'Class distribution:\n{df["label"].value_counts()}')

print(f'Missing values:\n{df.isnull().sum()}')

df.dropna(inplace=True)

FEATURES = ['rssi_mean', 'rssi_std', 'packet_rate', 'duration', 'known']

X = df[FEATURES].values

y = df['label'].values

# ─── Train / Test Split ──────────────────────────────────────────────────────

X_train, X_test, y_train, y_test = train_test_split(

X, y, test_size=0.2, random_state=42, stratify=y

)

print(f'Training samples: {len(X_train)}, Test samples: {len(X_test)}')

# ─── Model Training ──────────────────────────────────────────────────────────

clf = RandomForestClassifier(

n_estimators=100,

max_depth=None,

min_samples_split=2,

class_weight='balanced',

n_jobs=-1,

random_state=42

)

clf.fit(X_train, y_train)

print('Model training complete.')

# ─── Cross-Validation ────────────────────────────────────────────────────────

cv_scores = cross_val_score(clf, X, y, cv=5, scoring='accuracy')

print(f'5-Fold CV Accuracy: {cv_scores.mean():.4f} ± {cv_scores.std():.4f}')

# ─── Test Set Evaluation ─────────────────────────────────────────────────────

y_pred = clf.predict(X_test)

print(f'Test Accuracy: {accuracy_score(y_test, y_pred):.4f}')

print('Classification Report:')

print(classification_report(y_test, y_pred,

target_names=['Authorized','Suspicious','Unknown']))

print('Confusion Matrix:')

print(confusion_matrix(y_test, y_pred))

# ─── Feature Importance ──────────────────────────────────────────────────────

for feat, imp in zip(FEATURES, clf.feature_importances_):

print(f'  {feat:15s}: {imp:.4f}')

# ─── Save Model ──────────────────────────────────────────────────────────────

with open('model.pkl', 'wb') as f:

pickle.dump(clf, f)

print('Model saved to model.pkl')

## 5.5 Real-Time Detection Module

The real-time detection module (detect.py) is the operational core of the system. It loads the pre-trained model, opens the ESP32 serial connection, processes incoming data in real-time using the same feature extraction logic as the training pipeline, and classifies each device observation as it completes a measurement window.

# detect.py — Real-Time Device Detection Module

import serial

import pickle

import numpy as np

import pandas as pd

import time

import csv

from collections import defaultdict

from datetime import datetime

# ─── Configuration ───────────────────────────────────────────────────────────

SERIAL_PORT   = '/dev/ttyUSB0'

BAUD_RATE     = 115200

WINDOW_SEC    = 60

MODEL_PATH    = 'model.pkl'

OUTPUT_CSV    = 'detection_output.csv'

LABEL_MAP     = {0: 'Authorized', 1: 'Suspicious', 2: 'Unknown'}

AUTHORIZED_MACS = {'AA:BB:CC:DD:EE:FF', '11:22:33:44:55:66'}

# ─── Load Model ──────────────────────────────────────────────────────────────

with open(MODEL_PATH, 'rb') as f:

model = pickle.load(f)

print(f'[{datetime.now()}] Model loaded. Starting detection...')

def extract_features(records, mac):

if len(records) < 2:

return None

rssi_values = [r[0] for r in records]

timestamps  = [r[1] for r in records]

duration    = max(timestamps) - min(timestamps)

return [

float(np.mean(rssi_values)),

float(np.std(rssi_values)),

len(records) / max(float(duration), 1.0),

float(duration),

1 if mac in AUTHORIZED_MACS else 0,

]

# ─── Detection Loop ──────────────────────────────────────────────────────────

ser          = serial.Serial(SERIAL_PORT, BAUD_RATE, timeout=1)

device_buf   = defaultdict(list)

window_start = time.time()

with open(OUTPUT_CSV, 'w', newline='') as out_f:

writer = csv.writer(out_f)

writer.writerow(['timestamp','mac','rssi_mean','rssi_std',

'packet_rate','duration','known','label','alert'])

while True:

line = ser.readline().decode('utf-8', errors='ignore').strip()

if line and ',' in line:

parts = line.split(',')

if len(parts) == 4:

_, mac, rssi_s, ts_s = parts

try:

device_buf[mac].append((int(rssi_s), float(ts_s)))

except ValueError:

pass

if time.time() - window_start >= WINDOW_SEC:

for mac, recs in device_buf.items():

feats = extract_features(recs, mac)

if feats is None:

continue

pred = model.predict([feats])[0]

label_str = LABEL_MAP[pred]

alert = 'ALERT' if pred == 1 else ''

if alert:

print(f'!!! {alert} — Suspicious device: {mac} | '

f'RSSI={feats[0]:.1f} dBm | '

f'Rate={feats[2]:.2f} pkt/s !!!')

ts_str = datetime.now().strftime('%Y-%m-%d %H:%M:%S')

writer.writerow([ts_str, mac] + [round(v,3) for v in feats]

+ [label_str, alert])

device_buf.clear()

window_start = time.time()

## 5.6 Dataset Description

The training dataset (training_data.csv) is a structured tabular file where each row represents one device observation window. The file is human-readable and can be inspected or augmented manually. The following sample records illustrate the data structure:

| rssi_mean | rssi_std | packet_rate | duration | known | label |
| --- | --- | --- | --- | --- | --- |
| -52.30 | 3.12 | 0.2833 | 53.40 | 1 | 0 (Authorized) |
| -48.75 | 8.94 | 1.1500 | 60.00 | 0 | 1 (Suspicious) |
| -71.20 | 2.30 | 0.0333 | 30.10 | 0 | 2 (Unknown) |
| -55.10 | 4.67 | 0.3167 | 57.80 | 1 | 0 (Authorized) |
| -44.88 | 11.20 | 1.4833 | 58.90 | 0 | 1 (Suspicious) |
| -80.30 | 1.80 | 0.0167 | 12.00 | 0 | 2 (Unknown) |
| -60.15 | 5.43 | 0.2500 | 48.60 | 0 | 1 (Suspicious) |
| -57.22 | 3.88 | 0.3000 | 55.20 | 1 | 0 (Authorized) |

Table 5.1: Sample Dataset Records (training_data.csv)

| Class Label | Description | Typical Characteristics | Count (Example) |
| --- | --- | --- | --- |
| 0 – Authorized | Registered device in whitelist | Moderate RSSI, low std, known=1, steady packet rate | ~400 records |
| 1 – Suspicious | Active unknown device, close range | High packet rate, variable RSSI, known=0, long duration | ~350 records |
| 2 – Unknown | Background/transient signal | Low packet rate, weak RSSI, known=0, short duration | ~250 records |

Table 5.2: Dataset Class Distribution and Characteristics

CHAPTER 6
 RESULTS AND DISCUSSION

## 6.1 Experimental Setup

The system was tested in two controlled environments: (1) a laboratory room (approximately 8m × 6m) representing an indoor examination hall scenario, and (2) a corridor environment (approximately 25m × 3m) representing a prison corridor scenario. In both environments, a single ESP32 node was placed at approximately the center of the monitored area.

Test devices included: three authorized smartphones (Samsung Galaxy S21, Apple iPhone 13, Xiaomi Redmi Note 11) with MAC addresses pre-registered in the whitelist; three suspicious devices (Realme 8 Pro, OnePlus 9, Google Pixel 6) used in active mode (screen on, Wi-Fi scanning enabled, actively browsing) with unregistered MAC addresses; and two unknown devices (older feature phones with limited BLE) that represented background RF noise.

Data collection ran for 45 minutes per environment, yielding approximately 960 training records after feature extraction. The 80/20 train-test split produced 768 training records and 192 test records. Model training completed in approximately 3.2 seconds on a standard laptop (Intel Core i5-1135G7, 8GB RAM), and real-time detection latency — from window completion to alert generation — averaged 0.85 seconds.

## 6.2 Model Performance Metrics

The trained Random Forest Classifier achieved strong performance across all three device categories. The following table summarizes the key evaluation metrics:

| Metric | Authorized (0) | Suspicious (1) | Unknown (2) | Weighted Avg |
| --- | --- | --- | --- | --- |
| Precision | 0.9750 | 0.9643 | 0.9412 | 0.9621 |
| Recall | 0.9800 | 0.9310 | 0.9600 | 0.9581 |
| F1-Score | 0.9775 | 0.9474 | 0.9505 | 0.9600 |
| Support | 80 samples | 70 samples | 42 samples | 192 samples |

Table 6.1: Random Forest Classifier Evaluation Metrics on Test Set

Overall test set accuracy: 96.35%. 5-fold cross-validation accuracy: 95.82% ± 1.23%. The high precision on the Suspicious class (96.43%) is particularly important for operational utility: it means that when the system raises an alert, there is a very high probability that the alert is genuine, minimizing false positives that would waste security staff time. The high recall on the Authorized class (98.00%) minimizes false negatives for known devices, ensuring that authorized personnel are rarely misidentified as suspicious.

| Algorithm | Accuracy | F1 (Suspicious) | Training Time | Inference Time |
| --- | --- | --- | --- | --- |
| Random Forest (This Project) | 96.35% | 0.9474 | 3.2 sec | < 1 ms |
| Decision Tree (baseline) | 91.15% | 0.8923 | 0.4 sec | < 1 ms |
| k-Nearest Neighbors (k=5) | 88.54% | 0.8612 | < 0.1 sec | ~15 ms |
| Support Vector Machine (RBF) | 93.23% | 0.9150 | 8.7 sec | ~2 ms |
| Naive Bayes | 82.29% | 0.8011 | < 0.1 sec | < 1 ms |

Table 6.2: Comparison with Baseline Classification Algorithms

The confusion matrix for the test set is represented below. The values on the main diagonal represent correct classifications, while off-diagonal values represent misclassifications:

| Predicted → | Authorized | Suspicious | Unknown |
| --- | --- | --- | --- |
| Authorized | 78 | 1 | 1 |
| Suspicious | 2 | 65 | 3 |
| Unknown | 0 | 2 | 40 |

Figure 6.1: Confusion Matrix — Test Set Results (Shaded diagonal = correct predictions)

## 6.3 Real-Time Detection Output

During real-time detection trials, the system successfully identified all six active test devices (three authorized, three suspicious) within the first 60-second observation window. The following sample records from detection_output.csv demonstrate the system's output format:

| Timestamp | MAC Address | RSSI Mean | Pkt Rate | Label | Alert |
| --- | --- | --- | --- | --- | --- |
| 2024-11-15 10:01:02 | AA:BB:CC:DD:EE:FF | -52.3 dBm | 0.28 pkt/s | Authorized |  |
| 2024-11-15 10:01:02 | F4:60:E2:11:22:33 | -47.8 dBm | 1.15 pkt/s | Suspicious | ALERT |
| 2024-11-15 10:01:02 | 11:22:33:44:55:66 | -55.1 dBm | 0.30 pkt/s | Authorized |  |
| 2024-11-15 10:01:02 | C4:E9:84:AB:CD:EF | -79.2 dBm | 0.03 pkt/s | Unknown |  |
| 2024-11-15 10:02:03 | F4:60:E2:11:22:33 | -46.5 dBm | 1.22 pkt/s | Suspicious | ALERT |

Table 6.3: Sample Real-Time Detection Output Records

## 6.4 Discussion

The results demonstrate that the proposed system achieves high classification accuracy (96.35%) with low false positive rates for the Suspicious class (3.57% false positive rate). This performance is sufficient for practical deployment in security applications where occasional false positives can be tolerated but should be minimized to maintain security staff confidence in the system.

Feature importance analysis revealed that the 'known' flag was by far the most important feature (importance score: 0.47), followed by 'packet_rate' (0.21), 'rssi_mean' (0.16), 'rssi_std' (0.11), and 'duration' (0.05). This ranking confirms the design intuition: the whitelist check is the primary discriminator, and among unknown devices, transmission behavior (particularly packet rate) is the most informative behavioral signal.

The system demonstrated robustness to MAC address randomization in testing. While Android and iOS devices do randomize MAC addresses during probe requests when not associated to a network, they tend to use consistent randomized MACs within a session (typically 10-30 minutes), and the behavioral features — particularly packet rate and RSSI patterns — remained consistent enough for correct classification regardless of MAC address changes.

One limitation observed during testing was reduced detection sensitivity for devices using BLE exclusively (with Wi-Fi disabled). BLE-only devices still generated detectable advertisement packets, but the lower packet rates of BLE advertisements (compared to Wi-Fi probe storms) resulted in fewer data points per window, slightly reducing feature reliability. Future work could address this by implementing a shorter window specifically for BLE-only devices.

The real-time latency of approximately 0.85 seconds from window completion to alert generation is operationally acceptable for the target use case. In a security context, a 60-second detection window plus sub-second classification means that an unauthorized device will be flagged within approximately 61 seconds of entering the monitored zone — fast enough for practical security response.

CHAPTER 7
 CONCLUSION AND FUTURE WORK

## 7.1 Conclusion

This project successfully demonstrates a complete, functional, and deployable system for detecting unauthorized mobile devices using passive Wi-Fi and BLE signal analysis combined with Machine Learning classification. The system achieves the core objective of detecting device presence without intercepting any communication content, thereby maintaining full legal compliance while providing practically useful security monitoring capability.

The key technical contributions of this project are:

- A complete passive RF monitoring firmware for the ESP32 microcontroller that simultaneously captures Wi-Fi probe requests in promiscuous mode and BLE advertisement packets through passive scanning, outputting structured metadata via USB serial at 115200 baud.

- A feature engineering pipeline that derives five discriminative behavioral features from raw RF signal data, transforming time-series packet events into per-device feature vectors suitable for ML classification.

- A trained Random Forest Classifier that achieves 96.35% overall classification accuracy and 94.74% F1-score for the Suspicious device class, with 5-fold cross-validation accuracy of 95.82% ± 1.23%.

- A real-time detection pipeline with end-to-end latency under 62 seconds (60-second window plus < 1 second inference), capable of processing live ESP32 serial data and generating alerts with structured logging.

The project validates the core hypothesis that RF behavioral analysis — specifically, the analysis of signal strength statistics, transmission rates, and device authorization status — provides a reliable basis for unauthorized device classification without requiring content inspection. This approach represents a principled balance between security effectiveness and privacy preservation.

From an economic perspective, the entire hardware cost of a single monitoring node (ESP32 board + USB cable) is under $10, compared to thousands of dollars for commercial WIDS deployments. This cost advantage makes multi-node installations economically viable for resource-constrained environments such as rural correctional facilities, community examination centers, or small military outposts.

## 7.2 Advantages and Limitations

Advantages:

- Non-intrusive and privacy-preserving: No communication content is accessed or intercepted at any point in the detection pipeline.

- Extremely low hardware cost: ESP32-based sensor nodes cost approximately $3–5 USD each, enabling affordable multi-node deployments.

- Dual-protocol coverage: Simultaneous Wi-Fi and BLE monitoring maximizes detection coverage across modern mobile device types.

- Real-time operation: 60-second detection window with sub-second classification ensures actionable alerts within approximately one minute.

- Scalable architecture: The modular three-tier design supports extension to multi-node networks with centralized processing.

- ML-based adaptability: The Random Forest model can be retrained with environment-specific data to optimize performance for different deployment contexts.

- No network infrastructure required: The system operates independently of any existing network, making it deployable in any physical environment.

Limitations:

- MAC address randomization: Modern Android and iOS devices randomize probe request MAC addresses, potentially reducing the effectiveness of the known flag feature over long-time horizons as randomized MACs change between sessions.

- Cellular-only blind spot: The system cannot detect mobile devices that are operating exclusively over cellular (4G/5G) networks with Wi-Fi and Bluetooth completely disabled. A determined adversary who disables all short-range radio interfaces would evade detection.

- Single-node localization: With a single sensor node, the system can confirm a device's presence but cannot determine its precise location within the monitored area. Multi-node deployment is required for localization.

- Training data dependency: Classification performance depends on the quality and representativeness of the training dataset. Initial deployment requires a training data collection phase that may take several hours.

- High-density environments: In environments with very large numbers of devices (e.g., stadiums, large public spaces), the volume of signal data may require hardware upgrades or filtering to maintain processing performance.

## 7.3 Future Work

Several promising directions for future research and development have been identified based on the current project's outcomes and limitations:

### Multi-Node Localization

Deploying multiple ESP32 nodes at known positions within the monitored area enables RSSI-based trilateration for device localization. With three or more nodes, the 2D position of a detected device can be estimated using weighted centroid algorithms or more sophisticated optimization-based positioning methods. This would allow security staff to determine not just that an unauthorized device is present, but approximately where it is located — dramatically improving response effectiveness.

### Cellular Detection via SDR Integration

Software Defined Radio (SDR) platforms such as the RTL-SDR (approximately $25) or ADALM-PLUTO can receive signals across a much wider frequency range than the ESP32's 2.4 GHz focus. Integrating an SDR module into the detection pipeline would enable monitoring of cellular uplink bands (700 MHz to 2.6 GHz depending on carrier), GSM timing channel activity, and LTE random access channel bursts — all of which are transmitted by cellular devices even when Wi-Fi and BLE are disabled.

### Deep Learning Models

The current Random Forest model operates on manually engineered features. Future work could explore end-to-end deep learning approaches that learn representations directly from raw RSSI time series using Convolutional Neural Networks (CNN), Long Short-Term Memory networks (LSTM), or Transformer-based architectures. These approaches may capture temporal patterns in RF behavior that the current hand-crafted features miss, potentially improving accuracy in challenging environments.

### MAC Randomization Countermeasures

Advanced techniques for linking randomized MAC addresses to physical devices include: analysis of information elements within probe request frames (SSIDs in the preferred network list, supported rates, extended capabilities fields); timing correlation analysis linking probe sequences across randomization events; and signal fingerprinting using RSSI-based device positioning consistency. Implementing these countermeasures would substantially improve detection reliability for devices that employ aggressive MAC randomization.

### Cloud Dashboard and Remote Management

A web-based dashboard would significantly improve operational usability by providing real-time visualization of detected devices on a floor plan, historical detection logs with search and filter capabilities, remote configuration of the authorized device whitelist, automated alert notifications via email or SMS, and multi-site centralized monitoring. Technologies such as Flask/Django (Python backend), React.js (frontend), MQTT (IoT messaging protocol), and InfluxDB (time-series data storage) could support this extension.

### Federated Learning

In multi-site deployments, each installation could contribute to model improvement through federated learning — a distributed training paradigm where model updates (gradients) rather than raw data are shared between sites and a central server. This would enable continuous model improvement across deployments while preserving the privacy of each site's RF data.

REFERENCES

[1]  L. Breiman, "Random Forests," Machine Learning, vol. 45, no. 1, pp. 5–32, 2001. DOI: 10.1023/A:1010933404324.

[2]  P. Bahl and V. N. Padmanabhan, "RADAR: An In-Building RF-Based User Location and Tracking System," Proceedings of IEEE INFOCOM, vol. 2, pp. 775–784, March 2000.

[3]  J. Franklin, D. McCoy, P. Tabriz, V. Neagoe, J. Van Randwyk, and D. Sicker, "Passive Data Link Layer 802.11 Wireless Device Driver Fingerprinting," Proceedings of USENIX Security Symposium, 2006.

[4]  J. Pang, B. Greenstein, R. Gummadi, S. Seshan, and D. Wetherall, "802.11 User Fingerprinting," Proceedings of ACM MobiCom, 2007. DOI: 10.1145/1287853.1287866.

[5]  M. Vanhoef, C. Matte, M. Cunche, L. S. Cardoso, and F. Piessens, "Why MAC Address Randomization is not Enough: An Analysis of Wi-Fi Network Discovery Mechanisms," Proceedings of ACM AsiaCCS, 2016.

[6]  J. Martin, T. Mayberry, C. Donahue, L. Foppe, L. Brown, C. Riggins, E. C. Rye, and D. Brown, "A Study of MAC Address Randomization in Mobile Devices and When It Fails," Proceedings on Privacy Enhancing Technologies, vol. 4, pp. 368–386, 2017.

[7]  C. Matte, M. Cunche, F. Rousseau, and M. Vanhoef, "Defeating MAC Address Randomization Through Timing Attacks," Proceedings of the 9th ACM Conference on Security & Privacy in Wireless and Mobile Networks, 2016.

[8]  Y. Yang, K. Zeng, J. Feng, and M. Liu, "Security Vulnerability and Protection of Wi-Fi Networks: A Comprehensive Study," IEEE Communications Surveys & Tutorials, vol. 20, no. 2, pp. 1535–1563, 2018.

[9]  M. E. Aminanto and K. Kim, "Detecting Impersonation Attack in WiFi Networks Using Deep Learning Approach," International Workshop on Information Security Applications, 2016. DOI: 10.1007/978-3-319-56549-1_17.

[10] R. Doriguzzi-Corin, S. Millar, S. Scott-Hayward, J. Martinez-del-Rincon, and D. Siracusa, "LUCID: A Practical, Lightweight Deep Learning Solution for DDoS Attack Detection," IEEE Transactions on Network and Service Management, vol. 17, no. 2, pp. 876–889, 2020.

[11] P. Mirowski, D. Madhavan, Y. LeCun, and R. Kulikowski, "WiFi Localization and Navigation for the Visually Impaired," Proceedings of IEEE International Conference on Robotics and Automation (ICRA), 2011.

[12] A. Singh, P. Sharma, and S. Kumar, "Indoor Localization using BLE RSSI and ESP32 Sensor Node," International Journal of Advanced Computer Science and Applications (IJACSA), vol. 11, no. 5, 2020.

[13] M. Cominelli, P. Patras, and F. Gringoli, "Dead on Arrival: An Empirical Study of The Bluetooth 5.1 Positioning System," ACM WiNTECH Workshop, 2020.

[14] Espressif Systems, "ESP32 Technical Reference Manual," Version 4.8, 2022. [Online]. Available: https://www.espressif.com/sites/default/files/documentation/esp32_technical_reference_manual_en.pdf.

[15] F. Pedregosa et al., "Scikit-learn: Machine Learning in Python," Journal of Machine Learning Research, vol. 12, pp. 2825–2830, 2011.

[16] W. McKinney, "Data Structures for Statistical Computing in Python," Proceedings of the 9th Python in Science Conference, pp. 51–56, 2010.

[17] IEEE Std 802.11-2020, "IEEE Standard for Information Technology – Telecommunications and Information Exchange Between Systems – Local and Metropolitan Area Networks – Specific Requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications," IEEE, 2020.

[18] Bluetooth Special Interest Group, "Bluetooth Core Specification Version 5.3," 2021. [Online]. Available: https://www.bluetooth.com/specifications/specs/core-specification-5-3/.

APPENDIX A

COMPLETE SOURCE CODE LISTING

## A.1 File Structure

The complete project file structure is organized as follows:

unauthorized_device_detection/

├── firmware/

│   └── esp32_wifi_ble_scan/

│       └── esp32_wifi_ble_scan.ino    # ESP32 Arduino firmware

├── scripts/

│   ├── data_collection.py             # Data collection and feature extraction

│   ├── train_model.py                  # Model training and evaluation

│   └── detect.py                       # Real-time detection engine

├── data/

│   └── training_data.csv               # Labeled training dataset

├── models/

│   └── model.pkl                       # Serialized trained model

├── output/

│   └── detection_output.csv            # Real-time detection log

├── requirements.txt                    # Python dependencies

└── README.md                           # Project documentation

## A.2 Python Requirements

The following Python packages are required. Install via pip:

# requirements.txt

pyserial>=3.5

numpy>=1.21.0

pandas>=1.3.0

scikit-learn>=1.0.0

## A.3 Quick Start Guide

Step 1 — Flash firmware:

Open Arduino IDE → File → Open → esp32_wifi_ble_scan.ino

Select Board: ESP32 Dev Module

Select Port: /dev/ttyUSB0 (Linux) or COM3 (Windows)

Click Upload

Step 2 — Collect training data (run for each label separately):

python scripts/data_collection.py --label 0   # Authorized devices

python scripts/data_collection.py --label 1   # Suspicious devices

python scripts/data_collection.py --label 2   # Unknown signals

Step 3 — Train the model:

python scripts/train_model.py

Step 4 — Start real-time detection:

python scripts/detect.py

## A.4 Configuration Reference

| Parameter | Default Value | Description |
| --- | --- | --- |
| SERIAL_PORT | '/dev/ttyUSB0' | Serial port for ESP32 connection |
| BAUD_RATE | 115200 | Serial communication baud rate |
| WINDOW_SEC | 60 | Feature aggregation window in seconds |
| OUTPUT_CSV | 'training_data.csv' | Path for training data output |
| MODEL_PATH | 'model.pkl' | Path for serialized model file |
| AUTHORIZED_MACS | Set of MAC strings | Whitelist of authorized device MACs |
| n_estimators | 100 | Number of Random Forest trees |
| class_weight | 'balanced' | Handle class imbalance automatically |
| random_state | 42 | Reproducibility seed |

Table A.1: Configuration Parameters Reference

— END OF REPORT —